Sparsh Rai

Indian Institute of technology Kanpur

Off Grid Solar Powered EV Charging Station

EV Charging

Location-

- Azad Maidan(26.26’81’’N 80.32’28’’E)

- Elevation 120m

Specifications and Standards-

- Charging Capacity of 50KWh/day

- Level 2 charging

- 50Kw During Day time

- 2 Chargers of 10 Kw

- 5 Charger of 3 Kw

- 3 Chargers of 7Kw

- Bharat DC 001 (15Kw) Charger

- Max Output Voltage of 240 V for 4 wheeler

- Max Output Voltage of 48 for 3 Wheeler and 2 Wheeler

- Single Connector gun

- Solar Module Configuration

- AEG Solar

- 325Wp Max power

- 1.50m^2 Area

- Detailed Specification sheet is attached

- at the end

- Battery Specifications

- Li Ion Battery

- 25.6V Nominal Voltage

- C10 Capacity =100Ah

- DOD=95%

- 5 Years Battery Life

Load Requirement and Calculations

- This Charging station is designed to Handle 2400Kwh/day of charging in a whole day of which 50Kwh will be  during the day time and 50Kwh During the night time.

- Total consumption calculation

- Consumption  = 100 KWh/day

- Calculation of Solar Power plant according to MNRE:

- Total Consumption = 100 KWh/day

- Plant peak sunny hour operation = 4.5s (Normal consideration)

- Plant Performance = 70% = 0.7

- Solar Power plant capacity = Total consumption PSH X Plant Performance

= 1004.5*0.7 = 31.74 KW (Plant capacity to be installed)

- Plant Capacity = 32 KW

- Module Rating = 325 Wp

Total no of modules required = (32 x 1000 )/325 = 99

- Battery Sizing Calculation: (Li Ion Battery)

Battery will be required for night time only.

- Total consumption = 100 KWH

- Day of Autonomy = 2day (Means how many days it will carry the charge)

Margin is 15% extra i.e. (100% + 15%) = 115% =1.15 Battery unit is in AH

- PCU Charge controller Voltage 20 KW to 30 KW = 96 V

- Depth of Discharge (DOD) =Assume depth of discharge as 95 % = 0.95

- Battery Efficiency = 100 % = 1

- AH =Night time consumption X Design Margin X Day of AutonomyCharge controller Voltage X Depth of discharge X Battery Efficiency

AH = 100000 X 1 X 1.15 96 X 0.95 X 1 =  AH = 1260 AH So, we need 1260 AH Battery.

- Total storage of battery for 2 day = 2X96V X 1260

AH = 241KWH